EUROCONTROL AI Lab

Data description: datasets and fields

Prepared by Giuseppe Frau · September 2026 · Version 1.0

# 1. Overview

The AI Lab uses two public datasets. Both are pre-installed in the data/ folder of the course package, so no download is needed during the training. This document describes what each file contains, what every field means, the data-quality issues participants meet in the exercises, and the variables derived from the raw fields.

|  | Airport traffic | METAR observations |
| --- | --- | --- |
| Files | airport_traffic_2019.csv, airport_traffic_2023.csv, airport_traffic_2024.csv, airport_traffic_2025.csv | LIMC.csv |
| Provider | EUROCONTROL Aviation Intelligence Unit (AIU), Aviation Intelligence Portal | Iowa Environmental Mesonet (IEM), Iowa State University: ASOS/METAR archive |
| Content | Daily number of IFR arrivals and departures per airport | Hourly weather reports (METAR) at Milano Malpensa (LIMC) |
| One row is | one airport on one day | one METAR report |
| Coverage | 1 Jan 2019 – 31 Dec 2019 and 1 Jan 2023 – 31 Dec 2025; 330–377 airports in 42 states | 1 Jan 2020 00:50 UTC – 30 Dec 2025 23:50 UTC |
| Size | 487 687 rows, 13 columns, about 31 MB in total | 52 394 rows, 30 columns, about 9 MB |
| Used in | Exercise 1 | Exercises 2 and 3 |

# 2. Airport traffic dataset (EUROCONTROL AIU)

## 2.1 What it contains

For each airport and each calendar day, the file gives the number of IFR movements: flights operating under Instrument Flight Rules that depart from or arrive at the airport. VFR traffic (for example most general-aviation flights) is not counted. The counts are provided from two sources:

- Source 1, the EUROCONTROL Network Manager (columns ending in _1). It is available for every airport and every day in the file.

- Source 2, the airport operator (columns ending in _IFR_2). It is reported only by some airports and is empty in about 71 % of the rows.

The years 2020–2022 exist on the portal but were deliberately not included: the COVID-19 period would dominate a small forecasting exercise. 2019 is kept as a pre-COVID reference.

| Year | Rows | Airports | States |
| --- | --- | --- | --- |
| 2019 | 113 975 | 330 | 42 |
| 2023 | 114 115 | 331 | 42 |
| 2024 | 129 302 | 377 | 42 |
| 2025 | 130 295 | 375 | 42 |

## 2.2 Fields

| Field | Type / unit | Meaning | Missing | Used |
| --- | --- | --- | --- | --- |
| YEAR | integer | Year of the flight date | 0 % | – |
| MONTH_NUM | integer 1–12 | Month number | 0 % | – |
| MONTH_MON | text | Month abbreviation (JAN … DEC) | 0 % | – |
| FLT_DATE | date (YYYY-MM-DD) | Day the movements refer to | 0 % | Ex. 1 |
| APT_ICAO | text | ICAO code of the airport, e.g. EBBR = Brussels | 0 % | Ex. 1 |
| APT_NAME | text | Airport name | 0 % | – |
| STATE_NAME | text | State where the airport is located | 0 % | – |
| FLT_DEP_1 | flights / day | IFR departures, Network Manager source | 0 % | – |
| FLT_ARR_1 | flights / day | IFR arrivals, Network Manager source | 0 % | – |
| FLT_TOT_1 | flights / day | Total IFR movements (= departures + arrivals), Network Manager source. Target of Exercise 1. | 0 % | Ex. 1 |
| FLT_DEP_IFR_2 | flights / day | IFR departures, airport operator source | 71 % | – |
| FLT_ARR_IFR_2 | flights / day | IFR arrivals, airport operator source | 71 % | – |
| FLT_TOT_IFR_2 | flights / day | Total IFR movements, airport operator source | 71 % | – |

"–" means the field is not used in the exercises.

## 2.3 Data-quality notes

- FLT_TOT_1 always equals FLT_DEP_1 + FLT_ARR_1.

- Where both sources are available (139 airports), they agree closely: the median difference is about 0.5 %.

- Not every airport is present every day. Some airports join or leave the dataset during a year, which is why participants force a regular daily calendar before modelling.

- Some days show sharp drops caused by strikes and industrial action. At Brussels, for example, traffic fell to 71 movements on 13 February 2025 and to 195 on 31 March 2025, against a typical 500–650. The data does not flag these days; participants identify them in the exercise.

- The values range from 0 to about 1 700 movements per day. The busiest airports in 2025 are Istanbul, Amsterdam, London Heathrow, Paris CDG and Frankfurt (1 260–1 490 movements per day on average).

# 3. METAR observations at Milano Malpensa (IEM archive)

## 3.1 What it contains

A METAR is the standard aviation routine weather report issued by an airport's meteorological office. Malpensa issues one every hour at HH:50 UTC, plus occasional special reports when the weather changes significantly. The IEM archive decodes each report into columns and also keeps the original text in the metar column.

Important: the archive stores values in US units: °F, statute miles, inches of mercury and feet. The notebooks convert them to the units used in European aviation. Missing values are written as M.

### Example: one report decoded

LIMC 162250Z 02004KT 9999 -RA BKN015 13/13 Q1004 NOSIG

| Group | Meaning | Archive columns |
| --- | --- | --- |
| LIMC | Station: Milano Malpensa | station |
| 162250Z | Day 16 of the month, 22:50 UTC | valid |
| 02004KT | Wind from 020° at 4 kt | drct = 20, sknt = 4 |
| 9999 | Visibility 10 km or more | vsby = 6.21 (statute miles) |
| -RA | Light rain | wxcodes = -RA |
| BKN015 | Broken cloud (5–7 oktas) at 1 500 ft | skyc1 = BKN, skyl1 = 1500 |
| 13/13 | Temperature 13 °C, dew point 13 °C (saturated air) | tmpf = 55.4, dwpf = 55.4 |
| Q1004 | QNH 1004 hPa | alti = 29.65 (inHg) |
| NOSIG | No significant change expected (trend) | only in metar |

## 3.2 Fields

| Field | Unit | Meaning | Missing | Used |
| --- | --- | --- | --- | --- |
| station | text | ICAO code of the station (always LIMC) | 0 % | – |
| valid | date-time, UTC | Observation time | 0 % | Ex. 2, 3 |
| tmpf | °F | Air temperature. °C = (°F − 32) × 5/9 | < 0.1 % | Ex. 2, 3 |
| dwpf | °F | Dew point: the temperature at which the air would be saturated. Fog is likely when it is close to the air temperature. | < 0.1 % | Ex. 2, 3 |
| relh | % | Relative humidity, computed by IEM from temperature and dew point | < 0.1 % | Ex. 2 |
| drct | degrees true | Direction the wind blows from. Empty when the wind is calm or variable (VRB). | 19 % | Ex. 2, 3 |
| sknt | knots | Mean wind speed | < 0.1 % | Ex. 2, 3 |
| gust | knots | Wind gust, reported only when gusts occur | 99.7 % | – |
| vsby | statute miles | Prevailing visibility. 1 mile = 1 609.34 m. The maximum 6.21 mi corresponds to METAR 9999 (10 km or more). Basis of the Exercise 2 target. | < 0.1 % | Ex. 2, 3 |
| alti | inches of mercury | Altimeter setting (QNH). hPa = inHg × 33.8639 | < 0.1 % | Ex. 2, 3 |
| mslp | hPa | Sea-level pressure from the remarks section; not reported at LIMC | 100 % | – |
| p01i | inches | Precipitation in the last hour; not part of European METARs, always 0 here | 0 % | – |
| skyc1 … skyc4 | code | Cloud cover of layers 1–4 (lowest first); codes in section 3.4 | 50–100 % | idea |
| skyl1 … skyl4 | feet above ground | Height of the base of cloud layers 1–4 | 57–100 % | idea |
| wxcodes | code | Present weather, e.g. BR, FG, -RA; codes in section 3.4. Empty = no significant weather. | 81 % | – |
| feel | °F | Apparent ("feels like") temperature, computed by IEM | < 0.1 % | – |
| metar | text | The original METAR message | 0 % | shown |
| ice accretion (3 fields), peak wind (3 fields), snowdepth | various | Fields of US automated stations; not reported at LIMC | 100 % | – |

"shown" = displayed in the notebook but not used by the model; "idea" = mentioned among the optional ideas at the end of Exercise 2; "–" = not used.

## 3.3 Data-quality notes

- Reporting frequency: 52 298 routine reports at HH:50 and 96 special reports. The notebooks keep only the routine reports on a regular hourly grid; 294 hours (0.6 %) have no report.

- Visibility is capped: 81 % of the reports show the maximum value (10 km or more). The variable describes the low end precisely and says nothing about visibility above 10 km.

- Low visibility is rare: visibility is below 1 500 m in about 2.5 % of the hours, mostly in November–February, at night and around sunrise.

- Encoding errors: 4 reports contain physically impossible pressure values (900 and 2 000 hPa after conversion). The notebooks treat them as missing.

- Calm or variable wind: the empty drct values are not errors. They are coded in the METAR as 00000KT or VRB02KT, and the notebooks convert wind into east–west and north–south components so that calm wind becomes zero.

- Resolution: temperature and dew point are reported in whole °C in the METAR. The decimals in °F come from the conversion.

## 3.4 Codes

| Cloud cover code (skyc) | Meaning |
| --- | --- |
| FEW | Few: 1–2 oktas (eighths of the sky covered) |
| SCT | Scattered: 3–4 oktas |
| BKN | Broken: 5–7 oktas |
| OVC | Overcast: 8 oktas |
| VV | Vertical visibility: the sky is obscured (typically by fog), and skyl gives how far up one can see |
| NSC / NCD | No significant cloud / no cloud detected |
| empty | No cloud reported, for example under CAVOK (ceiling and visibility OK) |

| Weather code (wxcodes) | Meaning |
| --- | --- |
| BR | Mist: visibility reduced by water droplets, 1 000 m or more |
| FG | Fog: visibility below 1 000 m |
| BCFG / MIFG | Fog patches / shallow fog (ground fog) |
| FZFG | Freezing fog (temperature below 0 °C) |
| HZ | Haze (dry particles) |
| RA / DZ / SN | Rain / drizzle / snow |
| TS | Thunderstorm, for example TSRA = thunderstorm with rain |
| prefix - or + | Light or heavy intensity, e.g. -RA = light rain |

# 4. Variables derived in the exercises

## 4.1 Exercise 1: airport traffic

| Variable | Definition |
| --- | --- |
| Target | FLT_TOT_1 of the next day at the selected airport (EBBR by default) |
| Past traffic | FLT_TOT_1 of the previous L days (look-back window L, a hyperparameter: 3, 7, 14 or 28) |
| Weekday | Day of the week of the target day, one-hot encoded (7 values) |
| Split | Train 2023–2024 · validation Jan–Jun 2025 · test Jul–Dec 2025 |

## 4.2 Exercises 2 and 3: low-visibility nowcasting

| Variable | Definition |
| --- | --- |
| Target (proxy for LVP) | 1 if visibility (vsby) is below 1 500 m in any of the next three hourly reports (t+1 h, t+2 h, t+3 h), otherwise 0. Positive in about 3.6 % of the hours. The datasets contain no field recording LVP: this target is derived by the exercise and stands in for it. |
| temp_c, dew_c | Temperature and dew point in °C |
| spread_c | Temperature − dew point (°C); 0 means saturated air |
| rh | Relative humidity (%); in Exercise 3 it is recomputed from temperature and dew point (Magnus formula), so the service needs no extra input |
| wind_east, wind_north | East–west and north–south wind components (kt): sin/cos of direction × speed; 0 when calm |
| qnh_hpa | QNH in hPa, with impossible values removed |
| vis_m | Current visibility in metres, capped at 9 999 |
| hour_sin, hour_cos, month_sin, month_cos | Time of day and month on a circle, so that 23:00 is next to 00:00 and December next to January |
| Split | Train 2020–2023 · validation 2024 · test 2025 |

Why a proxy? Low-Visibility Procedures are declared by the aerodrome under local procedures, mainly on the basis of runway visual range (RVR) and cloud ceiling; the thresholds are airport-specific. Neither the declarations nor decoded RVR values are in the archive. Prevailing visibility below 1 500 m behaves like LVP (rare, winter and night, persistent) and is simple to explain, so it is used as a stand-in. RVR appears only inside the raw metar text (for example R35L/0300N, in about 4.5 % of the reports) and ceiling in skyc1/skyl1. An optional idea at the end of Exercise 2 suggests using them for a more LVP-like target.

# 5. Sources, licensing and how to re-download

| Dataset | Source and terms | Download |
| --- | --- | --- |
| Airport traffic | EUROCONTROL Aviation Intelligence Unit. Published for download and automated processing; no explicit licence is stated on the portal, so please confirm training use with AIU. Aggregated counts: no personal data. | One file per year from the CSV page: airport_traffic_YYYY.csv |
| METAR LIMC | Iowa Environmental Mesonet, Iowa State University. Free public archive of weather observations, no registration. No personal data. | ASOS/METAR download page: station LIMC, all variables, 2020-01-01 to 2025-12-31, comma-separated, missing = M |

Aviation Intelligence Portal: CSV datasets (ansperformance.eu/csv)

AIU Airport Traffic dataset reference (ansperformance.eu/reference/dataset/airport-traffic)

Iowa Environmental Mesonet: ASOS/METAR download (mesonet.agron.iastate.edu)